Highlight all lines that apply.

|  | MONTHLY | SPRING | FALL | ANNUAL |
| --- | --- | --- | --- | --- |
| ROOFING |  |  |  |  |
| Shingles damaged, loose, missing |  | ☺ | ☺ |  |
| Debris or branches on roof |  | ☺ | ☺ |  |
| Moss, mold, discoloration of roof surface |  |  | ☺ |  |
| Flat Roofs |  |  |  |  |
| Surface blisters, bubbling, cracking |  | ☺ | ☺ |  |
| Gravel eroded or missing |  | ☺ | ☺ |  |
| Flashings |  |  |  |  |
| Flashings leaking, missing, corroded, deteriorated or damaged |  | ☺ | ☺ |  |
| Eavestroughs and/or Downspouts |  |  |  |  |
| Blocked with debris |  | ☺ | ☺ |  |
| Loose, poor connections |  | ☺ | ☺ |  |
| Poor Slope |  | ☺ | ☺ |  |
| Galvanized aluminum gutters paint deteriorated |  |  | ☺ |  |
| Discharge or spillage close to house |  | ☺ | ☺ |  |
| Eaves |  |  |  |  |
| Soffits and fascia loose, damaged or rotted |  |  |  | ☺ |
| Paint deteriorated |  |  | ☺ |  |
| EXTERIOR |  |  |  |  |
| Doors and Windows |  |  |  |  |
| Caulking and weatherstripping intact |  | ☺ | ☺ |  |
| Glass panes broken or cracked |  | ☺ | ☺ |  |
| Storm screens installed |  |  | ☺ |  |
| Storm screens removed |  | ☺ | ☺ |  |
| Paint deterioration on sitis and/or frames |  | ☺ | ☺ |  |
| Wood rot on sills and/or frames |  | ☺ | ☺ |  |
| Debris in window wells |  | ☺ | ☺ |  |
| Exterior Walls |  |  |  |  |
| Mortar or masonry deteriorated or missing |  | ☺ | ☺ |  |
| Stucco loose or separating |  | ☺ | ☺ |  |
| Wood siding rot or damage |  | ☺ | ☺ |  |
| Paint deteriorated, blistering, bubbling |  | ☺ | ☺ |  |
| Siding damaged, loose, pieces missing |  |  | ☺ |  |
| Lot Area |  |  |  |  |
| Wood to earth close contact |  |  |  | ☺ |
| Overhanging or dead branches rubbing on roof |  |  | ☺ |  |
| Retaining walls rot or movement |  |  |  | ☺ |
| Sidewalk, driveway cracks and/or deterioration |  |  |  | ☺ |
| Uneven or sloped towards house |  |  |  | ☺ |
| Porches/Decks |  |  |  |  |
| Wood rot |  |  |  | ☺ |
| Paint problems |  | ☺ | ☺ |  |
| Debris under roof deck or porch |  |  | ☺ |  |
| Loose railings and/or steps |  | ☺ | ☺ |  |
| Garage |  |  |  |  |
| Evidence of movement |  |  |  | ☺ |
| Wood rot or deterioration |  |  |  | ☺ |
|  | MONTHLY | SPRING | FALL | ANNUAL |
| STRUCTURE |  |  |  |  |
| Foundation Walls |  |  |  |  |
| Deterioration or movement |  | ☺ | ☺ |  |
| Damp or efflorescence |  | ☺ | ☺ |  |
| Cracks |  | ☺ | ☺ |  |
| Walkout and floor drains clear of debris |  | ☺ | ☺ |  |
| Wood Framing |  |  |  |  |
| Wood rot |  |  |  | ☺ |
| Movement |  |  |  | ☺ |
| Wood Borers And Pests |  |  |  |  |
| Evidence of frass, bore holes |  | ☺ | ☺ |  |
| Evidence of wood rot or decay at structural components |  | ☺ | ☺ |  |
| Evidence of termite shelter tubes |  | ☺ | ☺ |  |
| Evidence of rodent droppings |  | ☺ | ☺ |  |
| Evidence of gnawing at wood, aluminum, mortar, household items, etc. |  | ☺ | ☺ |  |
| ELECTRICAL |  |  |  |  |
| Service Cable |  |  |  |  |
| Mast head and wires loose or frayed |  |  |  | ☺ |
| Caulking at wire to wall connection missing, damaged, deteriorated |  | ☺ | ☺ |  |
| Main Distribution Panel |  |  |  |  |
| Rust or water marks |  | ☺ | ☺ |  |
| Turn breakers on/off to ensure proper functioning |  | ☺ | ☺ |  |
| Fuses loose |  | ☺ | ☺ |  |
| Panel or fuses warm to the touch or buming smells |  | ☺ | ☺ |  |
| Distribution Wiring |  | ☺ | ☺ |  |
| Burnt or loose wiring |  | ☺ | ☺ |  |
| Frayed or damaged distribution wiring |  | ☺ | ☺ |  |
| Aluminum wire connections loose, missing |  | ☺ | ☺ |  |
| Fixtures and Receptacles |  |  |  |  |
| Covers on outside receptacles loose, damaged, missing |  | ☺ | ☺ |  |
| Receptacles loose, damaged, missing |  | ☺ | ☺ |  |
| Switches loose, damaged, missing |  | ☺ | ☺ |  |
| Improper bulb wattage in fixtures |  | ☺ | ☺ |  |
| Frayed or damaged extension or appliance cords |  | ☺ | ☺ |  |
| Ground Fault Interrupters |  |  |  |  |
| Test for proper function | ☺ |  |  |  |
| HEATING |  |  |  |  |
| All Central Systems |  |  |  |  |
| Inspected by a professional |  |  |  | ☺ |
| Electric Heat |  |  |  |  |
| Check fuses or circuit breakers |  |  | ☺ |  |
| Oil Furnaces and Boilers |  |  |  |  |
| Exhaust flue pipe loose or corroded |  |  | ☺ |  |
| Oil tank leakage, corroded |  |  | ☺ |  |
| Gas Furnaces and Boilers |  |  |  |  |
| Monitor for gas odours (on-going) | ☺ |  |  |  |
|  | MONTHLY | SPRING | FALL | ANNUAL |
| Forced Air System |  |  |  |  |
| Filters checked and cleaned or replaced as needed | ☺ |  |  |  |
| Humidifier water levels, check if clean | ☺ |  |  |  |
| Shut off water supply to humidifier |  | ☺ |  |  |
| Turn on water supply to humidifier |  |  | ☺ |  |
| Ductwork blocked or obstructed |  |  | ☺ |  |
| Electric Space Heat |  |  |  |  |
| Baseboard heater damaged, corroded, missing |  |  | ☺ |  |
| Adequate clearance from combustibles, |  |  | ☺ |  |
| COOLING AND HEAT PUMP |  |  |  |  |
| Inspect by a professional |  | ☺ |  |  |
| Condensate drain line leakage | ☺ |  |  |  |
| Condensor (exterior) off level, rust |  | ☺ |  |  |
| Freon lines damaged, missing, loose insulation |  | ☺ |  |  |
| Fan noisy, misaligned |  | ☺ |  |  |
| Window unit removed/protected |  |  | ☺ |  |
| PLUMBING |  |  |  |  |
| Water Supply System |  |  |  |  |
| Plumbing leakage, corrosion |  | ☺ | ☺ |  |
| Outdoor faucet shut off and drained |  |  | ☺ |  |
| Outdoor faucet turn on |  | ☺ |  |  |
| Main shut-off valve operational |  | ☺ | ☺ |  |
| Water Distribution System |  |  |  |  |
| Plumbing leakage, corrosion |  | ☺ | ☺ |  |
| Fixtures |  |  |  |  |
| Toilets, basins or faucets loose, damaged or leaking |  | ☺ | ☺ |  |
| Grouting and caulking mildew, mold, missing |  | ☺ | ☺ |  |
| Drainage System |  |  |  |  |
| Plumbing leakage, corrosion |  | ☺ | ☺ |  |
| Floor drains blocked with debris |  | ☺ | ☺ |  |
| Slow running drains |  | ☺ | ☺ |  |
| INTERIOR |  |  |  |  |
| Attic and Cathedral |  |  |  |  |
| Roof sheathing water stains, rot, mildew, fungus |  | ☺ | ☺ |  |
| Insulation wet, missing, damaged |  | ☺ | ☺ |  |
| Insulation over pot lights |  | ☺ | ☺ |  |
| Trusses and rafters wet, rot, damaged, movement |  | ☺ | ☺ |  |
| Vents obstructed |  | ☺ | ☺ |  |
| Evidence of rodents and pests |  | ☺ | ☺ |  |
| Walls and Ceilings |  |  |  |  |
| Drywall damaged, cracks, movement, bulging |  | ☺ | ☺ |  |
| Plaster separating, damaged | ☺ | ☺ |  |  |
| Door frames off-square, movement | ☺ | ☺ |  |  |
| Chimney |  |  |  |  |
| Parging and/or mortar loose and/or deteriorated |  | ☺ | ☺ |  |
| Chimney cap loose, missing, broken |  | ☺ | ☺ |  |
| Efflorescence on masonry |  | ☺ | ☺ |  |
| Professional inspection and cleaning of flue(s) |  |  |  | ☺ |
| Fireplaces and Stoves |  |  |  |  |
| Dampers inoperational, damaged, missing |  |  | ☺ |  |